SOP for ZY_EthovisionIO

Application: EthoVision-controlled Arduino I/O for a single behavioral chamber.

One-Time Setup

- Flash the Arduino firmware

  - Open ZY_EthovisionIO.ino in Arduino IDE and flash it to Arduino Uno R3.

- Optional: Test the Arduino

  - Open the Arduino IDE Serial Monitor

  - Test the connection by manually sending: p=13,v=1,d=5

  - Confirm that the command works correctly with pin13-LED on Arduino Uno R3

  - Close the Serial Monitor after testing.

- Optional: Rebuild the exe files (only needed if you want to modify the .cs source code)

  - Run build_UI.bat to recompile the .exe files.

Experimental Procedure

- Connect the Arduino

  - Connect Arduino to the PC and identify its COM port in Device Manager.

- Start ZY_AdapterUI

  - Launch ZY_AdapterUI.exe.

  - Select the correct COM port and connect: it should shows “Connected”.

  - Leave ZY_AdapterUI running.

- Configure EthoVision

  - In EthoVision, create an External Command Action.

  - Set Select program to run to: ZY_Controller.exe

  - Enter the desired command under Command line options (accepted pins 2-13).

Examples:

    - p=8,v=1 		→ turn Pin 8 ON

    - p=8,v=1,d=30.00 	→ turn Pin 8 ON for 30 seconds

    - p=8,v=0 		→ turn Pin 8 OFF

- Run the Experiment

  - Start the experiment in EthoVision.

  - Keep ZY_AdapterUI running throughout the session.

  - Monitor the Adapter log to confirm that commands are being received and executed correctly.

Shutdown

- Finish/stop the EthoVision experiment.

- Close ZY_AdapterUI.

- Disconnect the Arduino if needed.